语音导航实现

实验目的：

进一步熟悉机器人语音相关功能包的开发；

熟悉机器人地图构建与保存方法；

熟悉导航功能包的调用；

编程完成以下以下场景：

人：“元宝 元宝”

机器人：哎什么事儿？

人：“导航到上海馆”

机器人自主导航移动到上海馆

机器人到到达目标点后播报该地点的介绍语音比如：

机器人：“上海，中国的经济中心“。

实验原理：

分析实验任务我们需要做以下工作：

1 构建地图：

控制机器人在指定区域进行地图构建并保存地图，具体方法可以参考基础实验：机器人地图构建功能实现。

2标记坐标点：

在地图中标记出希望机器人到达的点位的坐标，具体方法可以参考基础实验：机器人多点导航功能实现。

3.程序流程如下所示：

调用：语音合成服务

输出音频

播放导航点介绍语音

返回顶部

调用：语音采集服务

调用：语音听写服务

采集一段音频

转化成为文字

判断文字中是否包含：”元宝元宝”

否

是

调用：语音合成服务

输出音频：哎,什么事儿？

调用：语音采集服务

调用：语音听写服务

采集一段音频

调用：导航控制机器人到达目标点

转化成为文字

判断文字中是否包含配置文件中地点的名字

是

等待机器人到达目标点

否

4 .需要打开的节点：

voice_collect_node：语音采集节点，启动后打开“voice_collect”服务。

voice_aiui_node：语音服务节点，该节点包括“/voice_iat”、“/voice_aiui”、“/voice_tts”、/voice_up_sync等服务。

5 .定义数据结构：

struct Point   //定义一个名为Student的结构体

{

float x;  //x坐标

float y; //y坐标

float z;//姿态z

float w;//姿态w

string name;//地点名字

string present;//介绍语

};

实验步骤：

- 创建实验文件，终端输入：

$ cd ~/ros_workspace/src/bobac3_audio/src

$ touch voice_nav.cpp

2 . 编写voice_nav.cpp代码：

#include <ros/ros.h>

#include <robot_audio/robot_iat.h>

#include <robot_audio/Collect.h>

#include <robot_audio/robot_tts.h>

#include <actionlib/client/simple_action_client.h>

#include "move_base_msgs/MoveBaseAction.h"

#include <iostream>

#include <string>

using namespace std;

struct Point   //定义一个名为Student的结构体

{

float x;  //x坐标

float y;  //y坐标

float z;  //姿态z

float w;   //姿态w

string name; //地点名字

string present; //介绍语

};

struct Point m_point[5]={{1.026,1.109,-0.012,1.000,"深圳","深圳，中国的科创中心"},{1.073,2.120,0.013,1.000,"上海","上海，中国的经济中心"},{2.48,0.13,0.03,0.99,"北京","北京，中国的首都"},{2.48,1.077,-0.026,1.000,"广州","广州，自古以来都是中国的商都"},{2.48,2.120,-0.032,0.999,"吉林","吉林，位于中国的东北是人参之都"}};

typedef actionlib::SimpleActionClient<move_base_msgs::MoveBaseAction> AC;//定义用到的action客户端的别名

class interaction{

public:

interaction();

string voice_collect(); //语音采集

string voice_dictation(const char* filename); //语音听写

string voice_tts(const char* text); //语音合成

void goto_nav(struct Point* point); //导航到目标位置

private:

ros::NodeHandle n; //创建一个节点句柄

actionlib::SimpleActionClient<move_base_msgs::MoveBaseAction>* ac; //创建action客户端对象指针

ros::ServiceClient collect_client,dictation_client,tts_client; //创建客户端

};

interaction::interaction(){

collect_client = n.serviceClient<robot_audio::Collect>("voice_collect"); //定义语音采集客户端

dictation_client = n.serviceClient<robot_audio::robot_iat>("voice_iat"); //定义语音听写客户端

tts_client = n.serviceClient<robot_audio::robot_tts>("voice_tts"); //定义语音合成客户端

}

string interaction::voice_collect(){

//请求"voice_collect"服务，返回音频保存位置

ros::service::waitForService("voice_collect");

robot_audio::Collect srv;

srv.request.collect_flag = 1;

collect_client.call(srv);

return srv.response.voice_filename;

}

string interaction::voice_dictation(const char* filename){

//请求"voice_dictation"服务，返回听写出的文本

ros::service::waitForService("voice_iat");

robot_audio::robot_iat srv;

srv.request.audiopath = filename;

dictation_client.call(srv);

return srv.response.text;

}

string interaction::voice_tts(const char* text){

//请求"voice_tts"服务，返回合成的文件目录

ros::service::waitForService("voice_tts");

robot_audio::robot_tts srv;

srv.request.text = text;

tts_client.call(srv);

string cmd= "play "+srv.response.audiopath;

system(cmd.c_str());

sleep(1);

return srv.response.audiopath;

}

void interaction::goto_nav(struct Point* point){ //导航到目标

ac = new AC("move_base",true);

ROS_INFO("Waiting for action server to start.");

ac->waitForServer();//一直等待move_base Action服务开启

ROS_INFO("Action server started, sending goal.");

//定义一个导航目标

move_base_msgs::MoveBaseGoal goal;

goal.target_pose.header.frame_id = "map";

goal.target_pose.header.stamp = ros::Time::now(); //设置时间戳

//导航点位置信息

goal.target_pose.pose.position.x = point->x;

goal.target_pose.pose.position.y = point->y;

goal.target_pose.pose.orientation.z = point->z;

goal.target_pose.pose.orientation.w = point->w;

ac->sendGoal(goal); //发送导航目标

ac->waitForResult(); //等待导航结果

if(ac->getState() == actionlib::SimpleClientGoalState::SUCCEEDED) //判断导航状态

ROS_INFO("Goal succeeded!");

ac->cancelGoal(); //取消动作

delete ac;

}

int main(int argc,char **argv){

ros::init(argc,argv,"interaction");

interaction audio; //创建一个交互实例

string dir,text,path; //创建两个字符串变量

while(ros::ok()){

dir = audio.voice_collect(); //采集语音

text = audio.voice_dictation(dir.c_str()).c_str(); //语音听写

if(text.find("元宝元宝") != string::npos){

audio.voice_tts("哎，什么事呀"); //合成应答语音

dir = audio.voice_collect(); //采集语音

text = audio.voice_dictation(dir.c_str()).c_str(); //语音听写

if(text.find("导航") != string::npos){ //识别到“导航”关键词

for(int i=0;i<5;i++){ //遍历所有参数

if(text.find(m_point[i].name.c_str()) != string::npos){ //查找所有导航点是否有匹配的导航点

audio.goto_nav(&m_point[i]); //导航到匹配的导航点

audio.voice_tts(m_point[i].present.c_str()); //介绍导航语

break;

}

}

}

}

}

return 0;

}

- 修改~/ros_workspace/src/bobac3_audio/src/CMakeLists.txt，添加如下：

add_executable(nav_node src/voice_nav.cpp)

add_dependencies(nav_node ${${PROJECT_NAME}_EXPORTED_TARGETS} ${catkin_EXPORTED_TARGETS})

target_link_libraries(nav_node

${catkin_LIBRARIES}

)

- 编译功能包，终端输入：

$ cd ~/ros_workspace

$ catkin_make

- 创建nav.launch启动文件，终端输入：

$ cd ~/ros_workspace/src/bobac3_audio/launch

$ touch nav.launch

内容如下所示：

<launch>

<!-- 实验节点 -->

<node pkg="bobac3_audio" type="nav_node" name="nav" output="screen"/>

<!-- 打开语音采集节点 -->

<node name="voice_collect" pkg="robot_audio" type="voice_collect_node" output="screen">

<!-- 音频文件目录 -->

<param name="audio_file" type="string" value="./AIUI/audio/audio.wav "/>

</node>

<!-- 打开语音服务 -->

<node pkg="robot_audio" type="voice_aiui_node" name="voice_aiui_node"/>

</launch>

- 开一个终端，打开机器人导航：

$ roslaunch bobac3_navigation demo_nav_2d.launch

- 打开终端，运行代码：

$ roslaunch bobac3_audio nav.launch

- 唤醒机器人，对机器人下达指令“导航到上海馆”：

- 机器人导航到目标位置：

- 到达位置后播放介绍语：